Anti-Oxidant Mixture protocol:

Materials:

- Glucose: 30 mg/ml in 20mM phosphate, 80 mM KCl, pH = 7.1

- Glucose oxidase: 20 mg/ml in 20mM phosphate, 80 mM KCl, pH = 7.4

- Catalyze: 3.5 mg/ml in 20mM phosphate, 50 mM KCl, pH = 7.4

Keep them in aliquots at -80C.

- DTT , 0.5 M in H2O.

Kept at -20.

Procedure:

- Thaw the tubes rapidly in your hand. Bring them to room temperature.

- Mix separately these two antioxidant solutions prior to use:

AO1: 10 uL. DTT + 10 uL. Glucose.

AO2: 10 uL Glucose Oxidase + 10 uL. Catalyze.

- Right before use, add 1/50 volume of each one (AO1 and AO2) to the final solution. (adjust this dilution  to optimize results, i.e.: 1/20, 1/30, 1/40)